# Solar Bot Wiring Guide

Updated to match the current project code and runtime configuration.

Hardware stack:

• Raspberry Pi 5

• Raspberry Pi Pico 2 W

• RadioMaster RP3 V2 receiver

• SmartElex u-blox GNSS GPS over USB

• MPU-6050 IMU

• L298N dual motor driver

• 12 V / 14.8 V motor battery, depending on your pack

# 1. Active Ports Used By The Code

| Device | Raspberry Pi port | Baud | Code/config |
| --- | --- | --- | --- |
| USB GPS | /dev/ttyACM0 usually, auto-detected | 9600 | mission_planner/gps_reader.py, config.yaml gps_port: auto |
| RadioMaster RP3 V2 | /dev/ttyAMA0 | 460800 | mission_planner/radio_bridge.py |
| Pico 2 W motor controller | /dev/ttyAMA4 | 115200 | mission_planner/robot_bridge.py |

The web app auto-start service runs:

/home/solar_bot/Desktop/Solar_bot-/.venv/bin/python -m mission_planner.app

Open the UI at:

http://192.168.2.6:5000

# 2. USB GPS To Raspberry Pi 5

Current working connection:

| GPS module | Raspberry Pi 5 |
| --- | --- |
| USB data/power cable | Pi USB port |

Expected Linux device:

/dev/ttyACM0

Useful checks:

lsusb

ls -l /dev/ttyACM0 /dev/serial/by-id/*

../.venv/bin/python test_gps.py --port /dev/ttyACM0 --seconds 10

Expected result:

FIX OK

Notes:

• Use a real USB data cable, not a charge-only cable.

• GPS LEDs only prove power/status. Linux must also show /dev/ttyACM0.

• The app config uses gps_port: auto, so USB GPS is preferred automatically.

Optional UART GPS wiring is not the current active setup. If used later, wire GPS TX to a Pi UART RX and update mission_planner/config.yaml.

# 3. RadioMaster RP3 V2 To Raspberry Pi 5

The mission planner expects MAVLink RC messages from the receiver:

/dev/ttyAMA0 @ 460800

Wiring:

| RadioMaster RP3 V2 | Raspberry Pi 5 | Notes |
| --- | --- | --- |
| TX | Pin 10, GPIO15 RX | Required, receiver data to Pi |
| RX | Pin 8, GPIO14 TX | Optional telemetry back to receiver |
| 5V | Pin 2 or Pin 4, 5V | Receiver power |
| GND | Pin 6, GND | Common ground |

Test:

../.venv/bin/python test_rp3v2.py

Mission planner mapping:

| Radio channel | Function |
| --- | --- |
| CH4 | Steering, left gimbal horizontal |
| CH2 | Throttle, right gimbal vertical |
| CH6 | Save boundary point |
| CH7 | Generate/start route |
| CH8 | Abort/reset safety action |

Important:

• The app uses MAVLink at 460800.

• If the receiver is configured for CRSF only, raw bytes may appear but the app will still show radio offline.

• The transmitter and receiver must be bound and outputting valid RC frames.

# 4. Raspberry Pi 5 To Pico 2 W UART

Current Pico firmware uses:

UART(0, baudrate=115200, tx=Pin(12), rx=Pin(13))

Wiring:

| Pico 2 W | Raspberry Pi 5 | Notes |
| --- | --- | --- |
| Pin 16, GP12 UART0 TX | Pin 33, GPIO13 RX | Pico STATUS data to Pi |
| Pin 17, GP13 UART0 RX | Pin 32, GPIO12 TX | Pi MOVE/STOP commands to Pico |
| GND | Any Pi GND, for example Pin 39 | Required common ground |

Pi UART device:

/dev/ttyAMA4 @ 115200

Required Pi config:

dtoverlay=uart4-pi5

Add it to:

/boot/firmware/config.txt

Then reboot.

Test Pico link:

../.venv/bin/python test_connection.py --port /dev/ttyAMA4

The app expects Pico telemetry lines like:

STATUS 0 0 <yaw> <speed_cms> <battery_mv> <wp_reached> <roll> <pitch>

If Pico is offline in the UI, battery and IMU will also show offline/0 because both come through this UART telemetry.

# 5. MPU-6050 IMU To Pico 2 W

The current firmware uses Pico I2C0:

I2C(0, sda=Pin(8), scl=Pin(9), freq=400000)

Wiring:

| MPU-6050 | Pico 2 W | Notes |
| --- | --- | --- |
| VCC | Pin 36, 3V3 OUT | Use 3.3 V |
| GND | Pin 38, GND | Common ground |
| SDA | Pin 11, GP8 | I2C0 SDA |
| SCL | Pin 12, GP9 | I2C0 SCL |
| AD0 | GND | I2C address 0x68 |
| INT | Pin 14, GP10 | Optional, not required by current code |
| XDA / XCL | Not connected | Not used |

The firmware checks for MPU-6050 at address:

0x68

The UI displays:

• IMU yaw

• Roll

• Pitch

• Tilt state

But only after the Pico sends valid STATUS lines to the Pi.

# 6. Pico 2 W To L298N Motor Driver

The current firmware pins are:

Left:  IN1=GP0, IN2=GP1, ENA=GP2 PWM

Right: IN3=GP3, IN4=GP6, ENB=GP7 PWM

Wiring:

| Pico 2 W | L298N | Function |
| --- | --- | --- |
| GP0, Pin 1 | IN1 | Left motor direction A |
| GP1, Pin 2 | IN2 | Left motor direction B |
| GP2, Pin 4 | ENA | Left motor PWM speed |
| GP3, Pin 5 | IN3 | Right motor direction A |
| GP6, Pin 9 | IN4 | Right motor direction B |
| GP7, Pin 10 | ENB | Right motor PWM speed |
| GND | GND | Common ground |

Power wiring:

| Power | L298N / Pico | Notes |
| --- | --- | --- |
| Battery positive | L298N 12V / VIN motor terminal | Motor power |
| Battery negative | L298N GND | Also tied to Pico GND |
| L298N 5V out | Pico VSYS | Only if the L298N 5V regulator is enabled and stable |
| L298N Motor A | Left motor wires | Swap wires if direction is reversed |
| L298N Motor B | Right motor wires | Swap wires if direction is reversed |

Important:

• For software PWM speed control, remove ENA/ENB jumpers and wire ENA to GP2, ENB to GP7.

• If ENA/ENB jumpers are installed, motors may run full speed and code speed control will not work correctly.

• All grounds must be connected together: Pi, Pico, L298N, receiver, and sensors.

Motor test:

../.venv/bin/python test_motors.py --port /dev/ttyAMA4 --speed 30

# 7. Battery Voltage Divider To Pico

Firmware ADC:

ADC(26)

Voltage divider:

Battery +  -> R1 100k -> GP26 ADC0

|

R2 33k

|

Battery - / GND ---------+

Wiring:

| Divider node | Pico |
| --- | --- |
| R1/R2 midpoint | GP26, ADC0 |
| Battery negative | Pico GND |

Notes:

• The divider scales battery voltage down for the Pico 3.3 V ADC.

• Do not connect battery positive directly to GP26.

• The UI battery percentage is 0 if Pico telemetry is missing.

# 8. Power And Ground Rules

Required common ground:

Raspberry Pi GND

Pico GND

L298N GND

Radio receiver GND

GPS GND through USB

Battery negative

Recommended power layout:

• Raspberry Pi 5: official/strong USB-C supply.

• Motors: separate battery into L298N motor input.

• Pico: USB during testing, or stable 5 V into VSYS.

• Radio receiver: Pi 5 V pin.

• GPS: Pi USB port.

Do not power motors from the Raspberry Pi 5.

If the Pi shows undervoltage warnings, USB GPS/radio/Pico links may disconnect.

# 9. Raspberry Pi 5 Setup

Enable UART4 for Pico:

sudo nano /boot/firmware/config.txt

Add:

dtoverlay=uart4-pi5

Then:

sudo reboot

Fix UART permissions:

cd /home/solar_bot/Desktop/Solar_bot-/solar_Bot

sudo bash fix_uart_permissions.sh

sudo usermod -aG dialout solar_bot

Log out/reboot after group changes.

# 10. Autostart Service

The project now auto-starts with:

solarbot-mission.service

Commands:

sudo systemctl status solarbot-mission.service --no-pager

sudo systemctl restart solarbot-mission.service

sudo systemctl stop solarbot-mission.service

journalctl -u solarbot-mission.service -f

If the service is running, do not also run python -m mission_planner.app manually, because port 5000 will already be in use.

# 11. Quick Verification Commands

GPS:

../.venv/bin/python test_gps.py --port /dev/ttyACM0 --seconds 10

Pico:

../.venv/bin/python test_connection.py --port /dev/ttyAMA4

Motors:

../.venv/bin/python test_motors.py --port /dev/ttyAMA4 --speed 30

Radio:

../.venv/bin/python test_rp3v2.py

Web app health:

curl http://127.0.0.1:5000/api/health

# 12. Expected UI Meaning

| UI item | Meaning |
| --- | --- |
| GPS good | USB GPS is connected and has fix |
| Pico offline | No valid STATUS lines from Pico on /dev/ttyAMA4 |
| Radio offline | No valid MAVLink RC frames from /dev/ttyAMA0 |
| Battery 0% | Pico telemetry missing or battery ADC reads 0 |
| IMU offline | Pico telemetry missing, or MPU-6050 not found |
| Green trail while idle | Should not happen after the GPS drift UI fix |

# 13. Full System Summary

USB GPS        -> Raspberry Pi 5 USB        -> /dev/ttyACM0 @ 9600

RP3 V2 TX      -> Pi GPIO15 RX, Pin 10      -> /dev/ttyAMA0 @ 460800

Pi GPIO12 TX   -> Pico GP13 RX, Pin 17      -> /dev/ttyAMA4 @ 115200

Pico GP12 TX   -> Pi GPIO13 RX, Pin 33      -> /dev/ttyAMA4 @ 115200

MPU-6050       -> Pico GP8/GP9 I2C0         -> 0x68

Pico GP0/1/2   -> L298N left motor control

Pico GP3/6/7   -> L298N right motor control

Battery divider-> Pico GP26 ADC0

L298N          -> Left and right DC motors